### Unify Openscape Business Phone System (Osbiz)

### CP200 TELEPHONE

Your CP200 telephone has a display to assist you.  If you like the option that is highlighted, press the    key.  If you want a different choice, use the      scroll key to move to the next option.

Preprogrammed keys on your CP200 telephone:-

The 4 keys at the top of the phone are often programmed as:

Incoming call answer and indication buttons (multiple)

Places calls on hold in a Park position (10 locations from 0 – 9).

Redials the last number called (press mutliple times for 2nd last call, 3rd last etc).

| FUNCTION | ###### DESCRIPTION |
| --- | --- |
| Phone Settings | To change the speaker volume of the handpiece. - Whilst in conversation, press + or – key near handpiece. To change the ringer volume. - With the handpiece in cradle press + button Screen: Telephone settings - Make sure “Ringer volume?” is highlighted on the screen. - Press: ##### Press + or – key until desired volume is reached. - Press: to save. To set the ringer tone. - With the handpiece in cradle press + button. Screen: Telephone settings - Scroll down so that “Ringer tone?” is highlighted on the screen. - Press: - Press + or – key until desired tone is reached. - Press: to save. |

| ### FUNCTION | ### DESCRIPTION |
| --- | --- |
| ### Internal Call | ### Lift handset and dial the extension number you require. |
| ### External Call | ### Lift handset dial 0 for outside line, then dial the number you require. |
| ### Answering Calls | ### If the phone is ringing, lift the receiver. |
| ### To Answer more than one call. | ### You will have to decide whether you would like to:- ### Transfer call you are attending to – refer below. ### Place call on hold – refer below. ### Place call in park – refer below. |
| ### (a) Transfer Call | - Make sure “Enquiry?” is highlighted on the screen. - Press key. - Key in extension number. - Press the transfer key. # If the extension you are transferring to is busy or not answering the call, you can return to the call. - Make sure “Return to held call?” is highlighted on the screen. # Press |
| ### (b) Hold | To put 1st caller on hold. - Make sure “Enquiry” is highlighted on the screen. - Press Hold key, then hang up (Your 1st caller is on hold and 1st Line/General Call key will flash slowly). You are now free to accept second call. To get your 1st caller back. - Press slow flashing Line/General Call key. |
| ### (c) Call Park | # To place call in park position. - Make sure “Enquiry” is highlighted # Press Park A Call key. # Enter and note park location (0-9). If park position number you entered is in use, enter another location. # Hang up the handset. # To retrieve a call from park position. # Press Park A Call key. # Enter park location number (0-9). |
| Handsfree Operation | - If speaking to caller using handpiece and you wish to use handsfree, press and hold Loudspeaker key until you place handpiece in cradle. |
| ### FUNCTION | DESCRIPTION |
| ### Call Back | If the internal extension you are trying to reach is busy or does not answer, you can store an automatic callback. - Make sure “Set Callback?” is highlighted on the screen. - Press  # To Cancel: # Key in #58 on keypad. # Screen: Callback : Ext no: # Make sure “Delete” is highlighted on the screen. # Press  |
| ### Programming DSS keys (For extensions or outside numbers.) | You can program your spare keys on the phone to be DSS keys for numbers you regularly call. - Press * then 9 then 1 and then the button you wish to configure. - Make sure “Change feature” is highlighted on the screen. - Press - Make sure “Repdial key?” is highlighted on the screen. - Press - Dial 0 for outside line then required number, or dial extension number if using an internal number. - Make sure “Save entry?” is highlighted on the screen. - Press - Make sure “End” is highlighted on the screen or scroll to Program another key. # Press |
| ### FUNCTION | DESCRIPTION |
| ### Voice Calling Announcement/ ### Intercom | This function allows you to issue a loudspeaker announcement to another internal extension. # Pick up handpiece. # Key in *80 on keypad. # Dial the extension number. # Speak the announcement. # The extension user hears a bleep tone followed by an announcement over the telephone loudspeaker. Extension user can reply but needs to make sure the Mute button is off (no light). |
| Do Not Disturb | Extension users can activate DND so that they can’t receive calls but can make calls. The main telephone should never be put into DND. To activate - *97. To cancel - #97. |
| Conference Calls | You are able to have a conference with several telephones at once. - Dial required internal extension, or 0 then number for an external number. Screen: Enquiry - Make sure “Enquiry” is highlighted on the screen. - Press - Dial 2nd required internal extension, or 0 then number for an external number. - When called party answers scroll down  to: “Conference”.  - Press If another party is required. - Make sure – Conference? is highlighted on the screen. - Press - Make sure “Add to party” is highlighted on the screen. - Press then dial 3rd required internal extension or 0 then number for an external number. - When called party answers, scroll down  to: “Conference”.  - Press |
| ### FUNCTION | DESCRIPTION |
| ### Individual Speed Dialling - ### Stores 10 numbers ### *0 to *9 | - Enter Code: *92 Screen: Abb. Dialing no: - Make sure “Next” is highlighted on the screen. - Press Screen: *0 - Scroll down so that “change entry” is highlighted on the screen. - Press Screen: *0: - Dial 0 – for outside line, then required number. - Make sure “Save entry” is highlighted on the screen. - Press Screen: *0: (number) Make sure “Next entry” is highlighted on the screen. - Press Screen: *1: - Scroll downso that “change entry” is highlighted on the screen. - Press - Dial 0 – for outside line, then required number. - Make sure “Save entry” is highlighted on the screen. - Press - Repeat above process until all required speed dial numbers have been entered. #### To change Individual Speed Dial numbers # Enter Code *92 Screen: Abb. Dialing no: - Make sure “Next” is highlighted on the screen. - Press Screen: *0: (number) - Make sure “Next” is highlighted on the screen if *0 is not the speed dial you wish to change and Press  - Repeat the last step until you reach the speed dial you wish to change. - Scroll downso that “Change entry” is highlighted on the screen. Press Screen: *0 - Dial 0 – for outside line, then required number. - Make sure “Save entry” is highlighted on the screen. - Press ###### To quit System Administration. Press Program/Menu Key. ###### To dial a number using Individual Speed Dialing. Code *7 then speed dial code (*0 through to *9). |
| FUNCTION | DESCRIPTION |
| Group Speed Dialing – stores numbers from 0001 to 9999 | To program group speed dials, it is necessary to start System Administration. This can only be done on the main phone. - Enter Code *95 Screen: System Administration user: - Press 1. - Press Screen: System Administration Ident/Password: - Press 7436367. - Press Screen: System admin. -User: 11 System speed dial 12 Time 13 Date - Make sure cursor is on 11 System speed dial. - Press Screen: 11 System speed dial 1 Speed dial number 2 Speed dial name Previous menu - Make sure cursor is on 1 Speed dial number. - Press Screen: 111 Abb. Speed dial number 000: + Next * Change - Scroll downto put cursor on * Change. - Press Screen: 111 Speed dial number Abb dial no: 000 F3 Delete Previous menu - Key in 0 for outside line then required number. - Make sure curser is on “Confirm”. - Press If entering more numbers:- Screen: 111 Abb. Speed dial number 000: (number) + Next * Change - Make sure curser is on “+ Next”. - Press - Scroll downto put cursor on * Change. - Press Screen: 111 Speed dial number Abb dial no: 001 F3 Delete Previous menu - Key in 0 for outside line then required number. - Make sure curser is on “Confirm”. - Press - Repeat above process until all required speed dial numbers have been entered. ###### To quit System Administration. Press Program/Menu Key. ###### To dial a number using Group Speed Dialing. Code *7 then speed dial code (0001 through to 9999). |
| ### FUNCTION | DESCRIPTION |
| ### Call Forwarding/ Divert | You can call forward your phone to another telephone. - With the handpiece in the cradle press the * key then 1 #### Screen: Diversion type: 1 = all calls 2 = only external calls 3 = only internal calls End - If you want to forward all calls make sure “1=all calls” is highlighted or scroll downso that your choice is highlighted on the screen. - Press Screen: all to: - Key in the extension number that calls are to be diverted to. Screen: all to: (number) - Make sure “Save entry” is highlighted on the screen. - Press To cancel Call Forwarding/Diversion - With the handpiece in the cradle press dial # then 1 |
| Change Time | You can change the system time and date but only on the main telephone. - Enter Code *95 Screen: System Administration User: - Press 1. - Press  Screen: System Administration Indent Password: - Press 7436367 - Press  Screen: System admin – User: 11 System speed dial 12 Time 13 Date - Scroll downto put cursor on “12 Time”. - Press Screen: 12 Time Format: hhmm: - Make sure the cursor is on “* Change”. - Press Screen: 12 Time Format hhmm: - Key in correct time in 24 hour format. - Make sure the cursor is on “Confirm”. - Press To quit System Administration. Press Program/Menu Key. |
| FUNCTION | DESCRIPTION |
| Call Transfer to ### Network (Note: This function needs to be programmed by a SATELCO Technician) | You can transfer a call to an external number. Screen: Enquiry - Press - Press a DSS key if one has been programmed for the required number, or dial 0 for outside line then required number. - Wait for the person to answer. - Press key. The call has now been transferred. If the person you were ringing does not answer and you wish to return to the caller: - Make sure “Return to held call” is highlighted on the screen. - Press You are now connected to your original caller. |
| Group Pick Up (Note: Pickup groups can be programmed by a SATELCO Technician) | If your extension is in a Pickup Group you can pick up calls for other extensions in your group. Screen: Call for: Pickup – Group - Press |
| Quick Reference Guide for Feature Codes Speed Number Dialling Call Forwarding Voicecall/Intercom Programming DSS keys Speed Number Programming Call Pick Up – Group Call Pick Up – Direct Clear unknown feature on extension Set Time and date Enable Call Waiting | Individual *7 then library number *0 to *9 Group *7 then library number 000 to 999 *11 *80 then individual extension *91 Individual *92 Group *95 *57 *59 then ringing extension number Lift handset #0 *95 user 1 / password 1 (only on main phone) #87 - *87 to disable |